ECA13 – MICROWAVE ENGINEERING

CO1 APPLICATION-BASED ASSESSMENT

Analysis, Design and Verification of an X-Band Microwave Transmission Line and Network

| Course | ECA13 – Microwave Engineering |
| --- | --- |
| Assessment | CO1 Application-Based Engineering Assignment |
| Student Name | Monisha Kumar |
| REG NO | 192412296 |

# ABSTRACT

This report develops and evaluates a microwave feed-line design for an X-band radar receiver/front-end. A printed 50 Ω microstrip line is selected because it is compact, low in mass, economical and directly integrable with PCB-mounted RF components. The nominal operating frequency is 10 GHz, with 8–12 GHz used as the evaluation band. The report formulates quantified requirements, compares coaxial cable, rectangular waveguide, stripline and microstrip using a weighted decision matrix, derives the transmission-line and quasi-TEM propagation model, estimates effective permittivity and propagation quantities, and develops a loss budget. It then represents the line as a two-port network, calculates load mismatch, discusses quarter-wave matching, and defines independent validation, tolerance, EMC and safety tests. The example uses a Rogers RO4003C-like substrate assumption (relative permittivity 3.55, thickness 0.508 mm and loss tangent 0.0027) and copper conductors. Analytical approximations are used where full-wave electromagnetic simulation or measured material data would be needed for definitive results.

# CONTENTS

- 1. Introduction and application specification

- 2. Stage 1 – Application and quantified requirements

- 3. Stage 2 – Transmission-structure evaluation and selection

- 4. Stage 3 – Mode and propagation model

- 5. Stage 4 – Loss budget and loss-reduction strategy

- 6. Stage 5 – Network model, matching and S-parameters

- 7. Stage 6 – Validation, critique, tolerance and test plan

- 8. Final engineering decision

- 9. Conclusion

- References

- Appendix A – MATLAB analytical verification code

# 1. INTRODUCTION

Microwave engineering concerns electromagnetic signals at frequencies where distributed effects, field structure, impedance discontinuities and material losses strongly affect circuit performance. At X-band, approximately 8–12 GHz, wavelengths are centimetre-scale in air and shorter in dielectric substrates. A few millimetres of line, a connector transition or a small geometry error can therefore alter phase and matching.

The selected application is the printed RF feed between an X-band radar receiver front-end and a low-noise amplifier/filter/mixer stage. The feed must transfer a weak received signal with low insertion loss, maintain a controlled impedance and avoid excessive reflections. Microstrip is selected because it supports compact PCB integration. It is not a pure TEM line: fields occupy both the substrate and air, so the effective permittivity is between 1 and the substrate relative permittivity and the mode is commonly treated as quasi-TEM when the line is electrically small in the transverse direction.

## 1.1 Historical context

Maxwell's equations established the unified electromagnetic-field model; Hertz experimentally demonstrated electromagnetic waves. The cavity magnetron and radar accelerated practical microwave engineering during the twentieth century. Later, satellite communication, solid-state microwave devices, monolithic microwave integrated circuits (MMICs), compact radar modules and high-frequency wireless systems made planar transmission lines central to modern RF design. The present design follows that progression by applying field and network theory to a manufacturable PCB feed.

## 1.2 Objectives

- Select and justify a real microwave application and a quantified operating specification.

- Compare transmission structures and their supported modes, losses, integration and shielding.

- Calculate microstrip effective permittivity, impedance and propagation quantities.

- Estimate conductor, dielectric, radiation, connector and mismatch losses and identify uncertainty.

- Develop a two-port network description, calculate reflection/VSWR/return loss and explain matching.  Validate the design against requirements and identify tests needed before hardware acceptance.

# 2. STAGE 1 – APPLICATION AND QUANTIFIED REQUIREMENTS (10 MARKS)

## 2.1 Application and category

Application: X-band radar receiver/front-end PCB feed. Category: civil/industrial sensing (the same RF principles also apply to weather and automotive radar). The feed connects a receiver module to a low-noise amplifier or filter stage. Because receiver sensitivity depends on preserving weak signals, insertion loss and impedance mismatch are important design constraints.

## 2.2 Band comparison

| Band | Approx. frequency | Free-space wavelength at representative frequency | Advantages | Limitations for this feed |
| --- | --- | --- | --- | --- |
| L | 1–2 GHz | ~20 cm at 1.5 GHz | Lower free-space path loss and less rain sensitivity | Larger components and antenna structures; less convenient for compact X-band radar |
| S | 2–4 GHz | ~10 cm at 3 GHz | Moderate size and established radar use | Larger feed/antenna dimensions than X-band |
| X | 8–12 GHz | ~3.0 cm at 10 GHz | Compact radar antennas and components; broad | Higher conductor/dielectric loss |
|  |  |  | 4 GHz evaluation band | and tighter fabrication tolerances than lower bands |
| Ku/Ka | 12–40 GHz (approx.) | ~1.7 cm at 18 GHz; ~8.6 mm at 35 GHz | Smaller apertures and wider potential bandwidth | Greater sensitivity to surface roughness, fabrication errors and atmospheric/rain effects at higher frequencies |

Selection: 10 GHz is chosen as the nominal design point within X-band. The 8–12 GHz interval is used as an evaluation band, not a claim that every radar application operates across the entire band. Final band allocation and emission limits must be checked against the intended country and application. 2.3 Quantified requirement specification

| Parameter | Design requirement | Acceptance rationale |
| --- | --- | --- |
| Application / category | X-band radar receiver PCB feed; civil/industrial sensing | A practical low-power receiver interconnect |
| Nominal frequency | 10 GHz | Representative X-band design point |
| Evaluation band | 8–12 GHz | Checks response away from the nominal point |
| Characteristic impedance | 50 Ω nominal; target 50 Ω ± 5% | Standard RF system interface |
| Maximum feed-line insertion loss | ≤ 0.50 dB for 50 mm line, excluding external receiver blocks | Keeps feed loss modest; final value must be checked by simulation/measurement |
| Return loss | ≥ 15 dB over evaluation band after matching | Limits reflected power |
| VSWR | ≤ 1.43 corresponding approximately to 15 dB return loss | Equivalent matching criterion |
| Isolation / shielding | Not assigned as an independent dB requirement for a single isolated trace; target crosstalk below −30 dB between adjacent RF paths | Must be measured on the assembled PCB if multiple paths are present |
| Signal power | Assumed receiver-level small signal; design target ≤ +20 dBm continuous until thermal qualification | Not a high-power transmitter feed; power handling must be verified |
| Board envelope | Line length 50 mm; substrate thickness 0.508 mm | Compact planar implementation |
| Cost | Use standard PCB fabrication and commercially available RF laminate; no numerical price asserted | Vendor/fabricator quotation is needed for a defensible cost |
| Environment | Normal indoor module; specify temperature range during detailed product design | Material properties vary with temperature and frequency |

## 2.4 EMI/EMC, safety and environmental constraints

The microstrip is unshielded on its top surface and can radiate or couple to nearby traces. A continuous ground plane, ground stitching vias near launches, short return-current paths, good connector grounding and adequate separation from digital clocks reduce coupling. The receiver feed is assumed to carry low power, so human exposure is not the primary design driver; nevertheless, the complete radar system must meet applicable RF exposure and emissions rules. The PCB should avoid unnecessary copper/laminate waste, and end-of-life disposal should follow electronic-waste requirements.

## 2.5 Acceptance criteria

The design is provisionally acceptable only if the manufactured line has approximately 50 Ω impedance, measured return loss of at least 15 dB after matching, insertion loss below 0.50 dB over the required band, no unwanted resonance in-band, and adequate crosstalk/EMC performance. Numerical loss estimates in this report are preliminary because radiation, launch and surface-roughness effects require detailed field simulation or measurement.

# 3. STAGE 2 – EVALUATE AND SELECT THE TRANSMISSION STRUCTURE (15 MARKS)

## 3.1 Mode and structure comparison

| Structure | Wave type / dominant mode | Strengths | Limitations for the selected PCB feed |
| --- | --- | --- | --- |
| Coaxial line | TEM; no cutoff for ideal fundamental mode. Higherorder modes arise above geometry-dependent limits. | Well shielded, flexible, controlled impedance, good connector ecosystem. | Needs cable/connectors and occupies volume; transitions to PCB can add loss and discontinuity. |
| Rectangular waveguide | TE10 dominant in standard rectangular guide; TE/TM modes have cutoff. | Low loss and high power handling over suitable bands. | Bulky relative to PCB traces; transitions and flanges complicate compact receiver integration. |
| Stripline | TEM in a homogeneous dielectric between ground planes. | Good shielding and low radiation; predictable field confinement. | Two ground planes increase layer/processing needs; dielectric loss remains. |
| Microstrip | Quasi-TEM at sufficiently low transverse dimensions relative to wavelength; fields occupy air and substrate. | Low cost, compact, accessible for components and planar integration. | More exposed fields, radiation/coupling, dispersion and roughness sensitivity. |

A hollow single-conductor waveguide cannot support TEM because TEM requires both electric and magnetic fields transverse to propagation and a nonzero electrostatic potential difference between at least two conductors. A hollow guide has only one conducting boundary and its allowed propagating solutions are TE or TM modes with cutoff.

Microstrip is quasi-TEM rather than pure TEM because the dielectric/air boundary makes the field distribution nonuniform across the transverse cross-section. A single effective permittivity approximates the phase behaviour; as frequency rises, dispersion and higher-order effects become more significant.

## 3.2 Weighted decision matrix

Scores use a 1–5 scale, where 5 is best. Weights total 1.00. Scores are engineering judgements for this specific low-power, compact PCB receiver feed; they are not universal rankings.

| Criterion | Weight | Coaxial | Waveguide | Stripline | Microstrip |
| --- | --- | --- | --- | --- | --- |
| Frequency / bandwidth | 0.15 | 4 | 5 | 5 | 5 |
| Loss | 0.20 | 3 | 4 | 4 | 4 |
| Power handling | 0.10 | 3 | 5 | 4 | 2 |
| Size / weight | 0.15 | 3 | 1 | 3 | 5 |
| Cost | 0.10 | 4 | 2 | 3 | 4 |
| Shielding / EMI | 0.10 | 5 | 5 | 5 | 2 |
| PCB integration | 0.20 | 2 | 1 | 3 | 5 |
| Weighted total | 1.00 | 3.25/5 | 3.10/5 | 3.80/5 | 4.10/5 |

Weighted scores: coaxial 3.25/5, rectangular waveguide 3.10/5, stripline 3.80/5 and microstrip 4.10/5. Microstrip is selected because integration, compactness and cost have high weights. Stripline is the strongest alternative where shielding and low radiation matter more than ease of access.

## 3.3 Sensitivity and frequency change

If shielding and low radiation are given much greater weights, stripline can overtake microstrip. If power handling and minimum transmission loss dominate, waveguide may rank first, especially when the mechanical envelope permits it. If the frequency is halved to 5 GHz, the microstrip remains usable but the same physical line is electrically shorter and conductor loss falls; a new impedance calculation is still required because effective permittivity and discontinuity behaviour vary with frequency. If frequency doubles to 20 GHz, the same line becomes more sensitive to conductor roughness, radiation, launch parasitics and dispersion; a higherperformance laminate and full-wave verification become more important. The waveguide alternative also changes its cutoff margin with frequency, so its cross-section must be reselected for the new band.

## 3.4 Design decision 1

For the specified application I select a 50 Ω microstrip line over a stripline because the receiver feed requires compact PCB integration, low fabrication complexity and direct placement of RF components. Stripline would be preferred if the dominant requirement were shielding and minimum radiation rather than accessible surface mounting. This selection is conditional on the measured loss and matching meeting the requirements.

# 4. STAGE 3 – MODE AND PROPAGATION MODEL (20 MARKS)

## 4.1 Telegrapher's equations

For a uniform line with per-unit-length resistance R, inductance L′, conductance G and capacitance C, Kirchhoff's laws give the distributed equations:

∂V(z,t)/∂z = −R I(z,t) − L′ ∂I(z,t)/∂t ∂I(z,t)/∂z = −G V(z,t) − C ∂V(z,t)/∂t For sinusoidal steady state using the e^(jωt) convention:

dV/dz = −(R + jωL′)I ;     dI/dz = −(G + jωC)V γ = α + jβ = √[(R + jωL′)(G + jωC)]

Z₀ = √[(R + jωL′)/(G + jωC)]

For a low-loss line, R ≪ ωL′ and G ≪ ωC, so β ≈ ω√(L′C), Z₀ ≈ √(L′/C), and α ≈ R/(2Z₀) + GZ₀/2. For microstrip these quantities are effective distributed parameters because fields occupy air and dielectric.

## 4.2 Maxwell wave equation and mode distinction

In a source-free, homogeneous, linear medium, Maxwell's curl equations are ×E = −jωμH and ×H = jωεE. ∇	∇ Taking the curl of the first equation and using ×H = jωεE gives × ×E = ω²μεE. With ·E = 0 in a ∇	∇ ∇	∇ homogeneous source-free region, ²E + k²E = 0, where k = ω√(με). For a uniform guide, fields vary ∇ longitudinally as e^(−jβz), giving a transverse eigenvalue relation β² = k² − k_c².

| Mode | Longitudinal field | Cutoff / dispersion | Wave impedance in a lossless |
| --- | --- | --- | --- |
|  |  |  | guide |
| TEM | Ez = 0 and Hz = 0 | No cutoff in a two-conductor TEM line; β = k in a homogeneous dielectric | E/H = η |
| TE | Ez = 0; Hz may be nonzero | fc = kc/(2π√με); β = √(k²−kc²) above cutoff | ZTE = ωμ/β |
| TM | Hz = 0; Ez may be nonzero | fc = kc/(2π√με); β = √(k²−kc²) above cutoff | ZTM = β/(ωε) |

Microstrip uses a quasi-TEM approximation: the dominant fields are largely transverse, but a small longitudinal field component exists because the cross-section is inhomogeneous. It does not have a simple rectangularwaveguide cutoff frequency for the fundamental quasi-TEM mode. The design instead checks impedance, dispersion and avoidance of substrate/packaging resonances and unwanted higher-order modes.

## 4.3 Microstrip dimensions and impedance calculation

Assumed substrate: relative permittivity εr = 3.55, thickness h = 0.508 mm, loss tangent tanδ = 0.0027; copper conductivity σ = 5.8 × 10⁷ S/m. Selected line width W = 1.10 mm and length ℓ = 50 mm. These are illustrative laminate assumptions and should be replaced with the exact fabricator's stack-up and frequency-dependent material data.

W/h = 2.165

εeff ≈ (εr+1)/2 + (εr−1)/2 [1/√(1+12h/W)] = 2.7735

Z₀ ≈ 120π / [√εeff (W/h + 1.393 + 0.667 ln(W/h + 1.444))] = 51.28 Ω

Calculated preliminary characteristic impedance is 51.28 Ω. This is close to 50 Ω but not a substitute for a field solver: copper thickness, solder mask, surface roughness, actual dielectric constant and launch geometry change the result. If the exact stack-up produces an out-of-tolerance impedance, tune W using a microstrip calculator or full-wave solver and then verify with a TDR/VNA.

## 4.4 Propagation parameters at 10 GHz

| Parameter | Formula | Calculated estimate |
| --- | --- | --- |
| Effective permittivity | εeff from microstrip approximation | 2.7735 |
| Phase velocity | vp = c/√εeff | 1.800e+08 m/s |
| Guide wavelength / guided wavelength | λg = vp/f | 18.001 mm |
| Phase constant | β = 2π/λg | 349.04 rad/m |
| Phase delay for 50 mm | τ = ℓ/vp | 0.278 ns |
| Phase shift for 50 mm | βℓ | 999.9° |
| Skin depth of copper | δ = √[2/(ωμσ)] | 0.661 μm |
| Copper surface resistance | Rs = √(πfμ/σ) | 26.090 mΩ/square |

## 4.5 Cutoff, single-mode range and higher-order modes

The fundamental microstrip quasi-TEM mode has no cutoff in the same sense as a hollow waveguide's TE10 mode. Therefore, reporting a rectangular-waveguide cutoff frequency for microstrip would be incorrect. The practical upper operating limit is instead determined by dispersion, substrate/ground-plane modes, radiation, discontinuities and the onset of higher-order field behaviour. For this reason, the intended operating range is provisionally set to 8–12 GHz and must be checked in a full-wave solver with the actual board dimensions, connector and enclosure.

A feed discontinuity can excite unwanted modes even when the straight uniform line behaves well. To reduce this risk: use a gradual connector launch, continuous ground plane, ground vias near the launch, avoid abrupt width changes, keep the line away from board edges and verify the assembled structure in a 3-D electromagnetic solver.

## 4.6 Design decision 2

The intended mode is the fundamental quasi-TEM mode. The calculated effective permittivity is 2.773, with a

guided wavelength of approximately 18.00 mm at 10 GHz. The fundamental mode has no waveguide-style cutoff; consequently, the dimensions are provisionally acceptable for a 10 GHz PCB feed only after confirming impedance, losses and unwanted-mode behaviour using the actual laminate stack-up and full-wave simulation.

# 5. STAGE 4 – LOSS EVALUATION AND LOSS-REDUCTION STRATEGY (15 MARKS)

## 5.1 Loss mechanisms

Total feed loss includes conductor loss, dielectric loss, radiation/coupling loss, connector/transition loss and mismatch loss. The first two are distributed losses along the line; connector and mismatch terms are discrete contributions. The following estimates are analytical and intended to show the calculation process. Radiation is especially geometry-dependent and is represented by a provisional allowance of 0.10 dB/m; a solver or measurement is required to establish it.

## 5.2 Skin depth and conductor loss estimate

δ = √[2/(ωμ₀σ)] = 0.661 μm at 10 GHz

Rs = √(π f μ₀/σ) = 26.090 mΩ/square

A simplified conductor attenuation estimate is αc ≈ Rs/(Z₀ Weff), with Weff ≈ W + 2h/π. This approximation is only a first-order estimate; a recognised microstrip conductor-loss expression or a full-wave solver is preferred for final design.

Weff ≈ W + 2h/π = 1.423 mm αc ≈ 8.686 Rs/(Z₀ Weff) = 3.105 dB/m

## 5.3 Dielectric loss estimate

A commonly used first-order microstrip dielectric attenuation estimate is applied here, with effective field filling represented by εeff and the substrate loss tangent. Because practical formulas differ in their field-filling approximation, the result should be treated as an estimate.

αd ≈ (πf/c) [εr/√εeff] [(εeff−1)/(εr−1)] tanδ   Np/m αd ≈ 3.643 dB/m

## 5.4 Loss budget

| Loss component | Initial estimate | Improved-design target | Notes |
| --- | --- | --- | --- |
| Conductor | 3.105 dB/m | 2.484 dB/m | Target assumes smoother copper / improved plating; must be verified |
| Dielectric | 3.643 dB/m | 2.550 dB/m | Target assumes a lower-loss laminate; requires material data |
| Radiation/coupling | 0.100 dB/m | 0.050 dB/m | Provisional target from improved launch, ground vias and layout |
| Distributed total | 6.848 dB/m | 5.084 dB/m | Sum of distributed terms |
| 50 mm line loss | 0.342 dB | 0.254 dB | Distributed attenuation × 0.05 m |
| Connector/transition allowance | 0.150 dB | 0.100 dB | Single combined allowance; validate with VNA/deembedding |
| Illustrative total incl. transition | 0.492 dB | 0.354 dB | Mismatch handled separately to avoid double counting |

Figure 1. Illustrative analytical attenuation contributions. The plot is calculated from the assumptions above and is not measured data.

## 5.5 Percentage contribution and dominant loss

| Mechanism | Estimated attenuation (dB/m) | Share of distributed total |
| --- | --- | --- |
| Conductor | 3.105 | 45.3% |
| Dielectric | 3.643 | 53.2% |
| Radiation/coupling allowance | 0.100 | 1.5% |
| Total | 6.848 | 100% |

The simplified estimate identifies conductor loss as the dominant term under the assumed dimensions and material properties. This conclusion is provisional: real microstrip attenuation can be strongly affected by copper roughness, plating, solder mask, dielectric dispersion and the feed transition.

## 5.6 Frequency trend and redesign

Figure 2. First-order estimated 50 mm line loss versus frequency over 8–12 GHz. Conductor loss increases approximately with √f through surface resistance, while dielectric loss generally increases approximately with f in this simplified model.

Loss-reduction strategy: use a low-loss RF laminate with verified Dk/Df, specify smooth copper or suitable lowprofile foil, maintain adequate trace width consistent with 50 Ω impedance, minimise line length and use a welldesigned RF connector launch with ground vias. The illustrative improved-design target reduces distributed conductor loss by 20%, dielectric loss by 30% and radiation allowance by 50%. These percentages are design targets rather than proven improvements; the actual dB improvement must be calculated from measured or solver-derived attenuation.

## 5.7 Design decision 3

The simplified model estimates conductor attenuation at 3.105 dB/m, dielectric attenuation at 3.643 dB/m and a provisional radiation allowance of 0.100 dB/m. The 50 mm distributed loss estimate is 0.342 dB, plus an assumed 0.15 dB connector/transition allowance. The target redesign is expected to reduce loss, but acceptance against the 0.50 dB requirement remains conditional on full-wave simulation and VNA measurement.

# 6. STAGE 5 – NETWORK MODEL, MATCHING AND S-PARAMETERS (20 MARKS)

## 6.1 Equivalent voltage and current

For TEM and quasi-TEM lines, voltage and current can be defined by cross-sectional integrals or quasi-static circuit quantities with good accuracy. For non-TEM waveguides, a unique scalar voltage and current are not generally defined because the fields are not uniquely represented by a single transverse electric potential. An equivalent waveguide transmission-line model can nevertheless be defined for a selected mode using modal voltage/current normalisation; the resulting equivalent impedance depends on the mode and field convention.

## 6.2 Incident and reflected waves

a = (V + Z₀ I)/(2√Re{Z₀}) ;     b = (V − Z₀* I)/(2√Re{Z₀})

For a real reference impedance Z₀, these definitions reduce to a = (V + Z₀I)/(2√Z₀) and b = (V − Z₀I)/(2√Z₀). At a port, S11 = b1/a1 with port 2 matched; S21 = b2/a1 with port 2 matched. S12 and S22 are defined analogously by exciting port 2. For a passive reciprocal two-port made from ordinary isotropic materials, S12 = S21 within measurement uncertainty.

## 6.3 ABCD matrix for a uniform line

[V₁  I₁]ᵀ = [[cosh(γℓ), Z₀ sinh(γℓ)], [sinh(γℓ)/Z₀, cosh(γℓ)]] [V₂  I₂]ᵀ

For an ideal lossless line, γ = jβ, so A = D = cos(βℓ), B = jZ₀ sin(βℓ), and C = j sin(βℓ)/Z₀. Cascaded sections are multiplied in order: Mtotal = M1 M2. For a matched 50 Ω line, S11 and S22 are ideally zero and S21 = S12 = e^(−γℓ).

## 6.4 Reflection, VSWR and return loss for a realistic load

To demonstrate mismatch, use a realistic illustrative load ZL = 75 Ω at the design frequency and the calculated line impedance Z₀ = 51.28 Ω.

ΓL = (ZL − Z₀)/(ZL + Z₀) = (75 − 51.28)/(75 + 51.28) = 0.1878

VSWR = (1+|Γ|)/(1−|Γ|) = 1.463

Return loss = −20 log₁₀|Γ| = 14.52 dB

This illustrative 75 Ω load does not meet the 15 dB return-loss target. A nominally matched 50 Ω load gives Γ = 0, VSWR = 1 and ideal infinite return loss; practical measured values are finite because connectors, line transitions, tolerances and losses are non-ideal.

## 6.5 Quarter-wave matching

A quarter-wave transformer between real positive impedances uses ZT = √(Zsource Zload) and electrical length λg/4 at the design frequency. For a 50 Ω system and a 75 Ω load:

ZT = √(50 × 75) = 61.24 Ω λg/4 at 10 GHz ≈ 4.500 mm

A transformer with the calculated impedance and quarter-wave electrical length ideally transforms 75 Ω to 50 Ω at 10 GHz. It is narrowband: the electrical length departs from 90° away from the centre frequency. Also, a 75 Ω microstrip width must be calculated from the same laminate stack-up rather than guessed. A tapered transition or multisection transformer may provide broader bandwidth across 8–12 GHz.

## 6.6 S-parameter interpretation and acceptance

| Parameter | Definition / relation | Design interpretation |
| --- | --- | --- |
| S11 | Input reflection coefficient with port 2 matched | Return loss = −20 log10¦S11¦; lower ¦S11¦ is better |
| S21 | Forward transmission coefficient | Insertion loss = −20 log10¦S21¦ for a passive line when power-wave conventions apply |
| S12 | Reverse transmission coefficient | S12 = S21 for a reciprocal network with consistent reference impedances |
| S22 | Output reflection coefficient with port 1 matched | Indicates output-port matching |
| Reciprocity | S12 ≈ S21 | Expected for a passive reciprocal line |
| Losslessness | S†S = I | Not expected for a real microstrip because conductor and dielectric losses exist |
| Symmetry | S11 ≈ S22 | Expected only if the physical two-port and terminations are symmetric |
| Matching | S11 ≈ 0 and S22 ≈ 0 | Ideal; acceptance target is return loss ≥ 15 dB |

For a line loss of 0.342 dB, the matched-line transmission magnitude is approximately |S21| = 10^(−IL/20) = 0.9613, ignoring launch and mismatch terms. The phase is approximately −βℓ; the 50 mm line contributes about 999.9 degrees at 10 GHz.

## 6.7 Network summary table

| Parameter | Before matching (75 Ω load) | After ideal quarter-wave match (10 GHz) | Target |
| --- | --- | --- | --- |
| Reference impedance | 51.28 Ω | 50 Ω system interface | 50 Ω nominal |
| Reflection coefficient magnitude | 0.1878 | 0 ideally | ≤ 0.178 for 15 dB return loss |
| VSWR | 1.463 | 1.00 ideally | ≤ 1.43 |
| Return loss | 14.52 dB | Ideal infinite; practical finite | ≥ 15 dB |
| Insertion loss | 0.342 dB distributed only | Line loss plus transformer/launch loss | ≤ 0.50 dB total feed target |
| Reciprocity | Expected for passive line | Expected | S12 ≈ S21 |
| Losslessness | No; real losses exist | No; matching does not remove line loss | Not required |

Verification method: implement the transmission line and quarter-wave transformer in MATLAB or a circuit simulator, then compare the analytical result with a Smith chart and a 2-port VNA measurement. No softwaregenerated S-parameter plot is claimed in this report; Appendix A provides reproducible analytical code for propagation and loss trends.

## 6.8 Design decision 4

The 75 Ω example load produces substantial reflection and does not meet the return-loss requirement. A quarterwave transformer is a valid narrowband compensation at 10 GHz, but full-band compliance over 8–12 GHz must be checked. For broad-band acceptance, use a tapered or multisection transformer and verify S11/S21 magnitude and phase over the complete operating band.

# 7. STAGE 6 – VALIDATION, CRITIQUE AND TOLERANCE (10 MARKS)

## 7.1 Specification verification

| Requirement | Specified | Initial analytical design | Final design target | Status |
| --- | --- | --- | --- | --- |
| Operating band | 8–12 GHz | Fundamental quasiTEM; band requires solver check | Verify no unwanted resonance across 8–12 GHz | Conditional |
| Characteristic impedance | 50 Ω ± 5% | 51.28 Ω by approximate formula | Tune trace width using exact stack-up; confirm TDR | Conditional |
| Insertion loss | ≤ 0.50 dB / 50 mm | Estimated distributed line loss plus assumed transition ≈ 0.492 dB | Measure with VNA and de-embed launches | Not yet verified |
| Return loss | ≥ 15 dB | 75 Ω test load gives 14.52 dB | Quarter-wave/taper match; VNA | Not yet verified |
|  |  |  | verification |  |
| VSWR | ≤ 1.43 | 75 Ω test load gives 1.463 | ≤ 1.43 across band | Not yet verified |
| Crosstalk / isolation | Below −30 dB between adjacent RF paths | Not computed analytically | Measure coupled paths / simulate layout | Not yet verified |
| Power / size | ≤ +20 dBm assumed; 50 mm line | No thermal simulation | Check temperature rise and laminate rating | Conditional |

## 7.2 Assumptions and their effect

- Constant εr: real laminate permittivity varies with frequency, temperature and fabrication lot, affecting impedance and phase delay.

- Idealised conductor: copper roughness, plating, finite thickness and current crowding can increase attenuation above the smooth-conductor estimate.

- Uniform cross-section: connectors, bends, pads and width changes cause discontinuities and local reflections.

- Radiation estimate: the assumed 0.10 dB/m allowance is not a universal material constant; it must be derived from full-wave simulation or measurement.

- Single effective permittivity: useful for first-order design but cannot fully reproduce dispersion and field details across a wide band.

- Connector loss: the 0.15 dB allowance is illustrative and depends on connector quality, launch design, soldering and calibration/de-embedding.

## 7.3 Dimensional tolerance estimate

A ±5% width tolerance gives W = 1.045–1.155 mm. Since microstrip impedance decreases as W increases, the narrow-width case has higher impedance and the wide-width case lower impedance. The approximate sensitivity can be evaluated by recalculating the microstrip formula at both extremes.

| Case | Width | Approx. impedance | Implication |
| --- | --- | --- | --- |
| Nominal | 1.100 mm | 51.28 Ω | Baseline |
| −5% width | 1.045 mm | 52.92 Ω | Narrow trace increases impedance |
| +5% width | 1.155 mm | 49.74 Ω | Wide trace decreases impedance |

This is a first-order tolerance estimate only; substrate thickness and dielectric constant also have manufacturing tolerances. A Monte Carlo analysis should vary W, h and εr together using the fabricator's stated distributions. If the resulting impedance spread exceeds 50 Ω ±5%, the nominal width should be retuned and the stack-up tolerance improved.

## 7.4 EMC and safety test plan

- Inspect the board for continuous ground, via placement, connector soldering and adequate separation from digital switching traces.

- Calibrate a vector network analyser with an appropriate calibration kit at the connector reference planes.

- Measure S11 and S21 across 8–12 GHz; use fixture de-embedding or a thru/line standard to separate launch loss from line loss.

- Measure a coupled adjacent-line coupon to check crosstalk against the −30 dB target.

- Use a spectrum analyser/near-field probe to identify unintended emissions if the assembled module shows interference.

- Operate at the intended maximum power while monitoring temperature; stop if the laminate or connector exceeds its rated limit.

- Repeat key measurements after temperature cycling and on multiple fabricated samples.

## 7.5 Further improvement

The most important unresolved limitation is that the current calculations do not include full-wave launch modelling or measured laminate data. The next design iteration should use the exact manufacturer stack-up in CST, HFSS or an equivalent solver, optimise the line width and matching taper, export S-parameters, and compare the simulation with a VNA coupon. This removes the main uncertainty while preserving the compact microstrip architecture.

# 8. FINAL ENGINEERING DECISION (10 MARKS)

## 8.1 Initial design: NO (not yet accepted)

The initial design is not accepted for final hardware release because its compliance is not yet demonstrated. The approximate impedance is near 50 Ω, but the 75 Ω illustrative load has only 14.52 dB return loss and a VSWR of 1.463, failing the specified matching targets. The total insertion-loss figure includes an assumed connector allowance and remains an estimate rather than a verified result.

## 8.2 Final design: CONDITIONAL YES after verification

The final concept is recommended conditionally: retain a 50 Ω microstrip feed on a low-loss RF laminate, optimise the launch and matching network, and accept it only after simulation and measurements confirm ≤0.50 dB insertion loss per 50 mm, return loss ≥15 dB across the required band, acceptable crosstalk, and stable impedance under manufacturing tolerances. A quarter-wave transformer is suitable for a single-frequency 10 GHz match; a tapered/multisection transformer is preferred if the full 8–12 GHz band must meet the target.

The selected structure is appropriate for the stated compact, low-power receiver application. It should not be selected without reconsideration for high-power transmission, severe EMI containment, or applications where the lowest possible attenuation is more important than board integration; stripline, coaxial cable or waveguide could then be superior.

# 9. CONCLUSION

The assignment has formulated a quantified X-band radar-feed specification and compared four transmission structures. A microstrip line was selected because it provides the best balance of compactness, cost and PCB integration for the assumed receiver application. The quasi-TEM approximation yielded an effective permittivity, impedance, guided wavelength and phase delay. A first-order loss budget identified conductor and dielectric losses as important distributed terms, while connector and mismatch effects require separate treatment. The network analysis showed how a 75 Ω load creates reflection and how a quarter-wave transformer can restore a match at the design frequency. Final adoption remains conditional on full-wave simulation, exact stack-up data and VNA validation; no unperformed simulation or measurement is represented as completed.

# REFERENCES

- D. M. Pozar, Microwave Engineering, 4th ed., John Wiley & Sons, 2012.

- R. E. Collin, Foundations for Microwave Engineering, 2nd ed., Wiley-IEEE Press, 2001.

- C. A. Balanis, Advanced Engineering Electromagnetics, 2nd ed., Wiley, 2012.

- D. K. Cheng, Field and Wave Electromagnetics, 2nd ed., Addison-Wesley, 1989.

- Rogers Corporation, RO4000 Series High Frequency Circuit Materials data sheet. Use the current manufacturer revision for the exact laminate selected.

- IEEE Std 145-2013, IEEE Standard for Definitions of Terms for Antennas, IEEE.

- IEC 61000 series, Electromagnetic compatibility (EMC)—relevant parts to be selected for the finished equipment and test environment.

- The assignment specification: ECA13 – Microwave Engineering, CO1 Application-Based Assessment, Academic Year 2027–28, supplied course document.

# APPENDIX A – MATLAB ANALYTICAL VERIFICATION CODE

The following MATLAB code reproduces the first-order effective-permittivity/impedance calculation and plots the estimated 50 mm line loss versus frequency. It is an analytical estimate, not a full-wave electromagnetic solver.

clc; clear; close all;

% ECA13 CO1 - X-band microstrip analytical estimate c = 299792458; mu0 = 4*pi*1e-7; er = 3.55;

h = 0.508e-3;          % substrate thickness (m)

W = 1.10e-3;           % trace width (m) L = 50e-3;              % line length (m)

tanD = 0.0027;

sigma = 5.8e7;          % copper conductivity (S/m)

u = W/h;

eeff = (er+1)/2 + (er-1)/2*(1/sqrt(1+12/u));

if u <= 1

Z0 = (60/sqrt(eeff))*log(8/u + u/4); else

Z0 = (120*pi)/(sqrt(eeff)*(u + 1.393 + ...

0.667*log(u + 1.444)));

end

fprintf('Effective permittivity = %.4f\\n', eeff); fprintf('Characteristic impedance = %.2f ohm\\n', Z0);

f = linspace(8e9,12e9,201); Weff = W + 2*h/pi; Rs = sqrt(pi*f*mu0/sigma); alpha_c = 8.686*Rs./(Z0*Weff);  % dB/m, first-order estimate alpha_d = 8.686*(pi*f/c).*(er/sqrt(eeff))* ...

((eeff-1)/(er-1))*tanD; % dB/m, approximation alpha_rad = 0.10;                % provisional allowance (dB/m) lineLoss = (alpha_c + alpha_d + alpha_rad)*L;

figure;

plot(f/1e9,lineLoss,'LineWidth',1.5); grid on;

xlabel('Frequency (GHz)'); ylabel('Estimated 50 mm line loss (dB)'); title('Analytical Microstrip Loss Estimate');

% Example load mismatch

ZL = 75;

GammaL = (ZL-Z0)/(ZL+Z0);

VSWR = (1+abs(GammaL))/(1-abs(GammaL)); ReturnLoss = -20*log10(abs(GammaL));

fprintf('75-ohm load: |Gamma| = %.4f\\n',abs(GammaL)); fprintf('VSWR = %.3f\\n',VSWR);

fprintf('Return loss = %.2f dB\\n',ReturnLoss);

% Quarter-wave transformer at 10 GHz

ZT = sqrt(50*75); fprintf('Quarter-wave transformer impedance = %.2f ohm\\n',ZT);